# Wren Robot Product Specification v1

Larkspur Logistics · Daniel Reyes · 05 September 2022

# Overview

Wren is an autonomous mobile picking robot for shelving up to 2.4 metres. It carries a single tote of up to 25 kilograms and navigates using lidar and ceiling-facing cameras, so it needs no floor markers or magnetic tape.

# Key specifications

Top speed is 1.5 metres per second, reduced to 0.8 metres per second within two metres of a person. Battery life is ten hours, with a 45-minute fast charge. Picking accuracy in lab tests is 99.2 percent.

Each Wren reports its position, battery and task queue to Harbor, our routing platform, every 200 milliseconds.

# Safety

Wren stops within 30 centimetres when a person steps into its path. Safety is certified against ISO 3691-4 for driverless industrial trucks. Every stop event is logged for review.

# Open questions

Narrow aisles under 90 centimetres remain a challenge. Gripping soft packaging such as bags of rice is unreliable in v1 and is planned for Wren 2.